Bharat Forge SNOP System Specification

1. System Overview & Strategic Context

1.1 Core Objective

Develop a Constraint-Aware Finite Capacity Planning System that synchronizes demand from various divisions with forging, heat treatment, and machining capacities across multiple configurable locations and plants. The system replaces spreadsheet-based planning with a real-time "Single Source of Truth" enabling CPPC to optimize for Revenue, Profit, and Utilization.

1.2 Manufacturing Context

- Primary Constraint: Forging capacity (dies are the bottleneck)

- Secondary Constraints:

  - Heat Treatment furnace availability

  - Machining line capacity

  - Raw material availability (grade-specific billets/bars procured against die codes, including lead-time, MOQ, and current stock position)

- Critical Asset: ~20,000 active dies with finite life (measured in "shots" or strokes)

- Planning Granularity: Line-level scheduling with die-level tracking

2. Organizational Hierarchy & Multi-Level Planning

2.1 Structural Topology

The organizational structure is fully dynamic and configurable via administrative interfaces. It supports complete CRUD operations at every level:

- Location (top level – e.g. Mundhawa, Baramati, Indapur, Chakan, …)

- Division (child of Location – e.g. FMD, MCD, Defence & Aerospace, CDFD NPD, …)

- Line (child of Division – e.g. FMD1, MCD7, … – the atomic planning & capacity unit)

No part of the application logic hard-codes specific locations, divisions, or line codes. All references use the configurable hierarchy IDs.

Hierarchy Rules:

- Every Line belongs to exactly one Division

- Every Division belongs to exactly one Location

- Planning, capacity definition, die assignment, scheduling, and most reporting are performed at the Line level

2.2 Planning Levels

| Level | Horizon | Frequency | Owner | Output |
| --- | --- | --- | --- | --- |
| AOP | 4 Years | Annual | Management | Strategic capacity targets |
| SNOP | Quarterly (3 months) | Weekly | CPPC | Rough-cut capacity plan |
| Master Production Schedule | 4 Weeks (Monthly) | Daily | CPPC Planner | Finite schedule by line/die |
| Execution | 48 Hours | Hourly | Plant PPC | Shop floor dispatch list |

3. Shift Operations & Capacity Calendar

3.1 Weekly Cycle Framework

Cycle Definition: Monday 07:00 AM to following Monday 07:00 AM (168-hour cycle) — configurable per Line if required.

3.2 Shift Structure

Shift patterns are defined per Line (or inherited from Division/Location defaults). Standard example (configurable):

| Shift | Timing | Duration | Crew Type | Primary Activities |
| --- | --- | --- | --- | --- |
| Shift A | 07:00-15:00 | 8 hours | Full skilled crew | Complex setups, die trials, NPD runs, high-precision parts |
| Shift B | 15:00-00:00 | 9 hours | Standard production | Standard runners/repeaters, automated lines |
| Shift C | 00:00-07:00 | 7 hours | Skeleton crew | Automated lines only (minimal changeover), lights-out manufacturing |

3.3 Capacity Calculation Parameters

Capacity is calculated and maintained per Line.

Available Hours per Line formula:

- Gross Hours = 168 (7×24) OR 120 (5×24) for non-continuous lines (configurable per Line)

- Effective Hours = Gross Hours − Planned Downtime − Changeover Overhead − Absenteeism (5%) (all values are configurable per Line)

365-Day Operations: Continuous operation is a Line-level setting. Changeover Sequencing Rules apply per Line.

3.4 Planned Downtime Management (Line Level)

The system provides full CRUD functionality for managing planned downtime at the individual Line level.

Downtime Record Attributes:

- Downtime ID (system-generated)

- Line (mandatory FK)

- Reason/Category (dropdown + free text): Planned Maintenance, Major Overhaul, Holiday/Shutdown, Energy Maintenance, Quality Audit, Training, Others

- Start Date & Time

- End Date & Time (or Duration + Duration Unit)

- Duration Unit: Hours / Days / Weeks / Months / Years

- Recurrence (optional): One-time / Daily / Weekly / Monthly / Yearly (with end date)

- Impact Type: Full Line Stoppage / Partial Capacity Reduction (%)

- Notes / Justification

- Created By, Approved By, Status (Draft / Approved / Cancelled)

- Audit Trail

CRUD Operations:

- Create, Update, Delete/Cancel (soft-delete), Bulk apply across Lines

Validations & Enforcement:

- Prevent overlapping downtime on same Line

- Scheduling Engine blocks production allocation during downtime windows

- MPS & Execution lists flag/exclude downtime periods

- Non-executable demand alerts if CDD conflicts with downtime

- Approval workflow for significant downtime (>24 h or >₹X impact)

Impact Visibility:

- Real-time Capacity Impact Dashboard (per Line / Division / Location)

- Effective capacity after downtime (% loss, hours lost)

- Utilization vs post-downtime capacity deviation alerts

- Gantt overlay: schedule vs downtime calendar

- What-If: simulate added downtime → see SNOP/MPS/revenue/OTIF impact

4. Die Management & Categorization System

4.1 Die Classification Matrix

| Category | Frequency | Planning Strategy | Inventory Policy | Changeover Priority |
| --- | --- | --- | --- | --- |
| Runner | High (Daily/Weekly) | Over-manufacturing allowed | 2-3 days buffer WIP | Priority 1 (SMED <30 min) |
| Repeater | Moderate (Monthly/Q) | Strictly demand-driven | Zero buffer (strategic only) | Priority 2 (block scheduling) |
| Stranger | Rare (1-2×/year) | On-demand only | No inventory | Priority 3 (maintenance windows) |

4.2 Die Master Data Attributes

- Die Code, Total/Remaining Life (shots), Refurbishment History, Status, Geometric Compatibility

- Assigned Line (mandatory FK)

- Associated Raw Material Code, Qty per Shot, Procurement Lead Time, MOQ, Alternate RM codes

4.3 Die Availability Calendar (DAC)

Die-level Gantt scoped to assigned Line, showing loading, changeover windows, conflicts, life alerts.

4.4 Raw Material Procurement & Inventory Linkage

(Unchanged – die → RM linkage, consumption at production confirmation on assigned Line)

4.5 Planned Die Downtime / Unavailability Management (New)

The system provides full CRUD functionality for managing planned unavailability periods at the individual Die level (parallel logic to Line downtime).

Die Unavailability Record Attributes:

- Unavailability ID (system-generated)

- Die (mandatory FK)

- Reason/Category (dropdown + free text): Refurbishment, Repair, Inspection, Trial/Testing Block, Coating/Heat Treatment, Storage/Relocation, Others

- Start Date & Time

- End Date & Time (or Duration + Duration Unit)

- Duration Unit: Hours / Days / Weeks / Months / Years

- Recurrence (optional): One-time / recurring patterns

- Impact Type: Full Unavailability / Reduced Shots per Hour (% efficiency loss)

- Notes / Justification / Work Order Reference

- Created By, Approved By, Status (Draft / Approved / Cancelled)

- Audit Trail

CRUD Operations:

- Create, Update, Delete/Cancel (soft-delete), Bulk apply to related dies (same part family / same Line)

Validations & Enforcement:

- Prevent overlapping unavailability on same die

- Scheduling Engine blocks allocation of that die during unavailability windows

- MPS & Execution lists exclude or flag jobs requiring unavailable dies

- Non-executable demand alerts if planned shots cannot be fulfilled due to die downtime

- Approval workflow for significant periods (>7 days or >₹X impact)

Impact Visibility:

- Die Availability Impact Dashboard (per die / Line / Division / Location)

- Remaining available shots after unavailability deduction

- Capacity loss (shots, tons, revenue potential)

- Deviation alerts when die utilization falls below target due to planned downtime

- Gantt overlay: production schedule vs die unavailability calendar

- What-If: simulate added die downtime → see impact on SNOP/MPS, bottlenecks, OTIF

Integration Points:

- Feeds into Die Availability Calendar (4.4), Capacity Management (6.3), Planning Engine (6.4)

- Visible in Scenario Workspace (6.5)

- KPI tracking: Die downtime variance (planned vs actual), Capacity loss due to die unavailability

5. Manufacturing Route Logic (Process Paths)

5.1 Route Definitions & Types

Routes define the sequence of operations a part must go through from raw material to finished goods / dispatch. Routes are defined in the Part Master and can have primary and alternate versions.

| Route Type | Typical Sequence | Typical Duration Basis | Typical Lead Time Range | Applicable Parts / Use Case | Outsource Option? |
| --- | --- | --- | --- | --- | --- |
| A – Raw Forging Dispatch | Raw Material → Forging → HT (if required) → Dispatch | Seconds (forging) → Hours (HT) | 1–7 days | Direct customer forgings, internal supply to external machining | Yes (vendor forging) |
| B – Forged + Machined Component | Raw Material → Forging → HT → Machining → Dispatch | Sec → Hrs → Minutes–Hours | 5–30 days | Most automotive & HCV components (crankshafts, axle beams, gears) | Yes (outsource machining) |
| C – High-Precision / Value-Added | Raw Material → Forging → HT → Processing (shot peening, coating, straightening, NDT) → Machining → Final Inspection → Dispatch | Sec → Hrs → Min–Hrs → Min–Hrs | 10–60 days | Defence, Aerospace, Turbo Machinery, Marine cert. parts | Partial (processing steps) |
| D – Forging Only (Internal Transfer) | Raw Material → Forging → HT → Transfer to another Division/Location | Sec → Hrs | 2–10 days | Inter-divisional supply (e.g. Mundhawa forging → Chakan machining) | Yes |
| E – NPD / Trial Route | Raw Material → Forging (trial die) → HT → Special Processing → Trial Machining → Validation | Variable (longer setup) | 30–180 days | New Product Development, First Article Inspection (FAI) | Yes (prototype vendors) |

Route Attributes (stored per Part Master):

- Route ID / Version

- Primary / Alternate flag

- Sequence steps (operation code, department, resource type: Line / Furnace / Machine group)

- Standard Cycle Time per operation (sec/pc or min/pc)

- Queue / Wait Time (configurable buffer)

- Transit Time between operations / locations

- Yield / Scrap Factor per step

- Approval required (QA hold points)

- Certification required (e.g. RINA, DNV, marine cert.)

5.2 Scheduling Direction & Logic

Backward Scheduling (preferred for firm demand):

- Start from Customer Delivery Date (CDD) or Required Date

- Subtract:

  - Final dispatch buffer

  - Final inspection / packing time

  - Machining / Processing cycle time + queue

  - HT cycle time + furnace load time

  - Forging cycle time + changeover

  - Raw material lead time

- Result: Required Start Date for forging / RM procurement

Forward Scheduling (used for feasibility check or capacity-constrained push):

- Start from today / current date

- Add cycle times, queues, transit, changeovers, Line/Die downtime

- Result: Projected Finish Date → compare with CDD

Buffer Logic (configurable per route/part):

- Safety buffer after forging (2–5 days for runners)

- Queue buffer before HT / Machining (1–3 days)

- Transit buffer between locations (1–2 days if inter-plant)

5.3 Constraint Integration during Routing

The scheduling engine respects:

- Line availability (shifts + planned Line downtime)

- Die availability (remaining life + planned Die unavailability)

- HT furnace capacity (kg/h, batch size, current load)

- RM availability & lead time

- Machining line / cell capacity (if route includes machining)

- Yield/scrap adjustment: demand quantity × (1 + cumulative scrap factor)

If any constraint is violated → demand flagged as non-executable (red in dashboard) with root-cause (e.g. “Die 4653 unavailable 15–20 Jan”, “No RM stock for grade EN8D”).

5.4 Cross-Operations Indent Propagation

Forging demand is the upstream driver.

Calculation Flow:

- Net Demand (Sales + Machining Indent + Processing Indent + Safety Stock) × (1.2 + Cumulative Scrap Factor per route)

- Forging shots required = Net Demand ÷ Pieces per shot

- RM Indent = Forging shots × RM Qty per Shot × (1 + RM Scrap Factor)

- If machining/processing required → downstream indent sent to MCD / Processing division (via internal interface)
This calculation setup should be configurable in system settings for Admin User

Indent Types:

- Machining Indent → sent to MCD Lines (P-Code based)

- Processing Indent → shot peening, coating, NDT, etc.

- Inter-location transfer indent (if forging & machining in different plants)

5.5 Exception Handling & Feasibility Checks

- Route Feasibility Check (before plan approval):

  - No active primary/alternate route → block

  - No compatible Line/Die for forging step → flag

  - RM lead time > available time to CDD → warning

- Alternate Route Auto-Switch (configurable rule):

  - If primary route blocked > X days → evaluate alternate

  - Approval required if cost/lead time increases > threshold

- Partial Fulfilment support: split demand across routes if capacity allows

6. Detailed Functional Requirements

6.1 Module: Demand Ingestion & Management

Demand enters the system exclusively through Excel sheets exported from SAP (no direct API, EDI, or customer portal integration in Phase 1).

Key Features & Workflow:

- Upload Mechanism:

  - Web-based file upload (drag-and-drop or browse)

  - Pre-defined Excel template (downloadable from system) with fixed columns:

    - SAP Order/Schedule Line Number

    - Material Code / P-Code

    - Customer / Division

    - Quantity

    - Requested Delivery Date (CDD)

    - Forecast flag (firm vs tentative)

    - Internal indent type (machining / processing / transfer)

    - Safety stock indicator (if applicable)

- Validation on Upload:

  - Mandatory fields, data types, positive quantities

  - Valid material codes (cross-check against Part Master)

  - Date format & logical checks (CDD > current date)

  - Duplicate / overlap detection (same line number)

  - Row-level error reporting (e.g. “Row 47: Invalid material code”)

- Staging & Approval:

  - Uploaded data lands in staging table

  - Planner reviews: accept/reject individual rows or entire file

  - After approval → merges into active demand pool

  - Versioning: each upload creates a timestamped snapshot

- Demand Categories & Prioritization:

  - Firm Sales Orders / Schedules

  - Forecast (tentative)

  - Internal Indents (machining, processing, inter-division)

  - Safety Stock Builds

  - NPD / Trial Orders

  - Priority ranking (configurable):

    - Aerospace / Defence

    - Export / Critical Customers

    - Domestic OEM / Tier-1

    - Aftermarket / Spares

    - NPD / Trial

- Netting & Adjustment:

  - Net demand = Gross demand − Available inventory − WIP − In-transit

  - Manual overrides allowed (with audit trail)

- What-If & Change Management:

  - Simulate demand increase / cutdown / shift

  - Immediate preview of revenue, profit, capacity, OTIF impact

  - Approval workflow: changes > ₹50 Lakh → Management; > ₹10 Lakh → Division Head

- Audit & Traceability:

  - Who uploaded what, when

  - Change history (before/after comparison)

6.2 Module: Inventory Visibility

6-stage multi-bucket tracking with full hierarchy drill-down.

Stages & Buckets:

- Raw Material

  - Available (free)

  - Allocated (reserved for planned orders/dies)

  - In Transit

  - Under Inspection / Quarantine

  - Consumed

  - Defective / Rejected

- Forged → Heat Treated → Processed → Machined → Finished Goods

  - Good / Defective / Godown / MRB

Core Capabilities:

- Real-time valuation (by grade, location, division, line, lot/batch, profit centre)

- Lot / batch traceability (RM lot → die → line → production order → finished part)

- Safety Stock Triggers:

  - Runners: <2 days cover → auto-alert & suggest inflation to 5 days

  - RM safety linked to die category (higher cover for runners)

- Atomic Synchronization Rule:

  - On production confirmation: RM consumption (Allocated → Consumed) + product QOH increase (good qty) happen together

  - On demand cutdown: release allocated RM + reduce projected QOH

- Excess / Obsolescence flagging (due to cutdowns or low demand)

- Aging analysis, expiry alerts (if applicable)

- Views:

  - Hierarchy drill-down (Location → Division → Line → SKU / Die)

  - Timeline / Gantt-style inventory projection

  - Excess / shortage heat-map

6.3 Module: Capacity Management

Capacity is defined, calculated, and maintained per Line.

Core Elements per Line:

- Gross Hours (168 for 7-day continuous / 120 for 5-day – configurable)

- Shift patterns & crew types (Shift A/B/C or custom)

- Planned Downtime calendar (integrated from 3.4)

- Changeover matrix (time by die family, runner/repeater/stranger)

- Holiday & shutdown calendar

- Absenteeism factor (default 5%, configurable)

- Buffer for unplanned downtime (≤10%)

Targets & Overrides:

- Default targets:

  - Runners: 85–95% utilization

  - Repeaters: 70–80%

  - Strangers: Gap-filling

  - Overall: 80–85%

- Overrides possible at Division or Location level

- Supplier lead-time integration for external constraints (future)

Additional Features:

- Bottleneck identification (visual highlighting)

- Capacity release / booking for maintenance / NPD blocks

- Real-time effective capacity recalculation after downtime updates

6.4 Module: Planning & Allocation Engine (Core)

Scheduling & allocation happen at Line + Die level.

Hard Constraints (must be satisfied):

- Die remaining life ≥ required shots

- Die unavailability periods (no allocation during refurbishment/repair)

- Line availability (shifts + planned downtime)

- Tonnage, shut height, press compatibility

- HT furnace kg/h & batch size constraints

- Raw material availability (allocated stock + procurement lead time)

- Route feasibility (primary/alternate)

- Labor / crew availability (if modelled)

Soft Constraints / Optimization Objectives:

- Minimize changeovers (prefer SMED runners)

- Maximize contribution margin (revenue – variable cost)

- Load balancing across lines

- Minimize WIP & queue times

- Prioritize dies with sufficient RM stock

- Favor runners/repeaters over strangers

- Minimize RM excess / obsolescence from cutdowns

Algorithms:

- Primary: Constraint Programming (CP) solver

- Optimization pass: Genetic Algorithm (GA)

- Real-time adjustments: Greedy heuristic + manual override

Output & UI Behaviour:

- Color-coded Gantt chart:

  - Green = Runner

  - Yellow = Repeater

  - Red = Stranger

  - Blue = NPD / Trial

- Drag-and-drop rescheduling

- Real-time conflict/shortage highlighting

- Hover tooltips: die life remaining, RM status, violated constraints

- Feasibility status per demand line (executable / partially / blocked + reason)

6.5 Module: Scenario Planning Workspace

Purpose: Allow planners to simulate multiple “what-if” versions of the plan without affecting the live schedule.

Core Capabilities:

- Create unlimited scenarios from current base plan

- Modifications via drag-drop or bulk edit:

  - Change demand quantities / dates

  - Add/remove Line or Die downtime

  - Shift priorities or routes

  - Adjust capacity parameters

- Real-time recalculation of all KPIs:

  - Revenue, profit, contribution margin

  - Capacity utilization (%)

  - OTIF projection

  - RM stock-out risk

  - Bottleneck count

- Side-by-side comparison views (Gantt delta, KPI delta tables)

- Visual alerts: conflicts, shortages, overloaded lines

- Save, name, share, lock scenarios

- Export (PDF/Excel) for review / approval

- Approval workflow to promote a scenario to live plan



6.6 Module: Shop-Floor Feedback Loop & MES Integration (Closed-Loop Planning)

- Purpose Enable structured, real-time/periodic capture of actual shop-floor execution data directly via system forms to close the planning loop. This updates inventory, die life, capacity models, master data (cycle times, yields, scrap factors), and generates variance analysis for continuous improvement. It creates a learning, adaptive system that improves forecast accuracy, reduces plan-to-actual gaps, and drives root-cause analysis for downtime, scrap, and utilization deviations.

- Scope & Assumptions

- Data ingestion exclusively via web-based forms entered by floor planners, supervisors, or operators (no Excel uploads for feedback; demand ingestion remains Excel-based).

- Forms are role-based (e.g., operator view simplified; supervisor can approve/override).

- Accessible on shop-floor devices (tablets/PCs)